3-switch hierarchical VLAN topology where:

- SW1 is the Core Switch.

- SW2 and SW3 are Access Switches.

- Each switch hosts 2 departments (VLANs).

- The router performs inter-VLAN routing using Router-on-a-Stick.

# Topology

Router1
                      |
                   SW1 (Core)
                  /         \
                 /           \
               SW2           SW3

            #TSG             #MIS
            #ACCOUNTING      #SORTING
            #PST             #PRINTING

# VLAN Plan

| VLAN ID | Department | Network | Gateway |
| --- | --- | --- | --- |
| 10 | TSG | 192.168.10.0/24 | 192.168.10.1 |
| 20 | ACCOUNTING | 192.168.20.0/24 | 192.168.20.1 |
| 30 | PST | 192.168.30.0/24 | 192.168.30.1 |
| 40 | MIS | 192.168.40.0/24 | 192.168.40.1 |
| 50 | SORTING | 192.168.50.0/24 | 192.168.50.1 |
| 60 | PRINTING | 192.168.60.0/24 | 192.168.60.1 |

# SW1 (Core Switch)

### Create VLANs

enable
conf t

vlan 10
 name TSG

vlan 20
 name ACCOUNTING

vlan 30
 name PST

vlan 40
 name MIS

vlan 50
 name SORTING

vlan 60
 name PRINTING

### Department Ports

interface fa0/1
 switchport mode access
 switchport access vlan 10

interface fa0/2
 switchport mode access
 switchport access vlan 20

### Trunk Ports

interface gi0/1
 switchport mode trunk

interface gi0/2
 switchport mode trunk

interface gi0/3
 switchport mode trunk

Gi0/1 -> Router1
Gi0/2 -> SW2
Gi0/3 -> SW3

# SW2

### Create VLANs

vlan 10
vlan 20
vlan 30
vlan 40
vlan 50
vlan 60

### Department Ports

interface fa0/1
 switchport mode access
 switchport access vlan 30

interface fa0/2
 switchport mode access
 switchport access vlan 40

### Trunk to Core

interface gi0/1
 switchport mode trunk

# SW3

### Create VLANs

vlan 10
vlan 20
vlan 30
vlan 40
vlan 50
vlan 60

### Department Ports

interface fa0/1
 switchport mode access
 switchport access vlan 50

interface fa0/2
 switchport mode access
 switchport access vlan 60

### Trunk to Core

interface gi0/1
 switchport mode trunk

# Router1 Configuration

enable
conf t

interface g0/0/0
 no shutdown

interface g0/0/0.10
 encapsulation dot1Q 10
 ip address 192.168.10.1 255.255.255.0

interface g0/0/0.20
 encapsulation dot1Q 20
 ip address 192.168.20.1 255.255.255.0

interface g0/0/0.30
 encapsulation dot1Q 30
 ip address 192.168.30.1 255.255.255.0

interface g0/0/0.40
 encapsulation dot1Q 40
 ip address 192.168.40.1 255.255.255.0

interface g0/0/0.50
 encapsulation dot1Q 50
 ip address 192.168.50.1 255.255.255.0

interface g0/0/0.60
 encapsulation dot1Q 60
 ip address 192.168.60.1 255.255.255.0

# Example PC Addressing

### SW1

TSG PC
IP: 192.168.10.10
GW: 192.168.10.1

ACCOUNTING PC
IP: 192.168.20.10
GW: 192.168.20.1

### SW2

PST PC
IP: 192.168.30.10
GW: 192.168.30.1

MIS PC
IP: 192.168.40.10
GW: 192.168.40.1

### SW3

SORTING PC
IP: 192.168.50.10
GW: 192.168.50.1

PRINTING PC
IP: 192.168.60.10
GW: 192.168.60.1

# Verification

show vlan brief

show interfaces trunk

show ip interface brief

Expected trunk links:

SW1 Gi0/1 -> Router
SW1 Gi0/2 -> SW2
SW1 Gi0/3 -> SW3
SW2 Gi0/1 -> SW1
SW3 Gi0/1 -> SW1

Expected result:

- All 6 VLANs are carried across trunks using 802.1Q.

- Each department is isolated in its own VLAN.

- The router allows communication between departments.

- PCs in TSG, ACCOUNTING, PST, MIS, SORTING, and PRINTING can all reach each other through Router1.