# VLAN Configuration and Router-on-a-Stick Lab Notes

## 1. Topic Studied

Inter-VLAN Routing using VLANs, Trunk Links, and Router-on-a-Stick Configuration

The goal of the lab was to separate departments into different VLANs and allow communication between them using a router.

# 2. VLAN Overview

A VLAN (Virtual Local Area Network) divides a physical switch into multiple logical networks.

Benefits:

- Improves security

- Reduces broadcast traffic

- Separates departments

- Makes network management easier

Example VLAN structure:

| VLAN ID | Department |
| --- | --- |
| 10 | TSG |
| 20 | ACCOUNTING |
| 30 | PST |
| 40 | MIS |
| 50 | SORTING |
| 60 | PRINTING |

# 3. Creating VLANs on a Switch

Example:

enable
configure terminal

vlan 20
name ACCOUNTING

vlan 50
name SORTING

vlan 60
name PRINTING

end

Verify VLANs:

show vlan brief

# 4. Assigning Switch Ports to VLANs

Access ports connect end devices such as PCs.

Example on Switch03:

configure terminal

interface fa0/1
switchport mode access
switchport access vlan 60

interface fa0/2
switchport mode access
switchport access vlan 50

interface fa0/3
switchport mode access
switchport access vlan 20

end

Verification:

show vlan brief

Expected result:

VLAN 20  ACCOUNTING  Fa0/3
VLAN 50  SORTING     Fa0/2
VLAN 60  PRINTING    Fa0/1

# 5. Trunk Configuration

A trunk carries multiple VLANs between switches or between a switch and router.

Example:

interface fa0/4
switchport mode trunk

Check trunk status:

show interfaces trunk

A working trunk should show:

Status: trunking
Allowed VLANs: 1,10,20,30,40,50,60

# 6. Router-on-a-Stick Configuration

A router uses subinterfaces to provide gateways for each VLAN.

The physical interface:

GigabitEthernet0/0/0

Subinterfaces created:

| VLAN | Gateway |
| --- | --- |
| 10 | 192.168.10.1 |
| 20 | 192.168.20.1 |
| 30 | 192.168.30.1 |
| 40 | 192.168.40.1 |
| 50 | 192.168.50.1 |
| 60 | 192.168.60.1 |

Example:

interface gi0/0/0.10
encapsulation dot1Q 10
ip address 192.168.10.1 255.255.255.0

Repeat for other VLANs.

Verification:

show ip interface brief

Successful output:

Gi0/0/0.10  192.168.10.1  up/up
Gi0/0/0.20  192.168.20.1  up/up
Gi0/0/0.30  192.168.30.1  up/up
Gi0/0/0.40  192.168.40.1  up/up
Gi0/0/0.50  192.168.50.1  up/up
Gi0/0/0.60  192.168.60.1  up/up

# 7. PC IP Configuration

Each PC needs:

### Accounting PC (VLAN 20)

IP Address: 192.168.20.2
Subnet Mask: 255.255.255.0
Default Gateway: 192.168.20.1

### Sorting PC (VLAN 50)

IP Address: 192.168.50.2
Subnet Mask: 255.255.255.0
Default Gateway: 192.168.50.1

### Printing PC (VLAN 60)

IP Address: 192.168.60.2
Subnet Mask: 255.255.255.0
Default Gateway: 192.168.60.1

# 8. Troubleshooting Commands

## Check VLANs

show vlan brief

## Check trunks

show interfaces trunk

## Check router interfaces

show ip interface brief

## Test connectivity

From a PC:

ping 192.168.x.1

Example:

ping 192.168.20.1

# 9. Problems Found During the Lab

### Problem 1:

Configured:

interface fa0/0.10

Error:

Invalid interface type and number

### Cause:

The router did not have a FastEthernet 0/0 interface.

### Solution:

Use the correct interface:

interface gi0/0/0.10

### Problem 2:

Switch03 VLANs existed but no ports were assigned.

Before:

20 ACCOUNTING active
50 SORTING active
60 PRINTING active

No interfaces were shown.

Solution:

Assign PC ports:

switchport access vlan <number>

# 10. Final Network Checklist

✅ VLANs created
✅ PC ports assigned to VLANs
✅ Switch-to-switch links configured as trunks
✅ Switch-to-router link configured as trunk
✅ Router subinterfaces created
✅ PC IP addresses configured
✅ Default gateways configured
✅ Ping tests successful

Main concept learned:
VLANs separate networks, trunks carry multiple VLANs, and router subinterfaces allow communication between different VLANs.