# VLAN Trunking & Router-on-a-Stick Lab

## Topology

PC01 --- SW0 --- SW1 --- PC03
PC02 --- SW0 SW1 --- PC04
|
Router1

### VLAN Assignment

| Device | VLAN | IP Address |
| --- | --- | --- |
| PC01 | 10 | 192.168.10.10/24 |
| PC03 | 10 | 192.168.10.20/24 |
| PC02 | 20 | 192.168.20.10/24 |
| PC04 | 20 | 192.168.20.20/24 |

### Default Gateways

- VLAN 10 → 192.168.10.1

- VLAN 20 → 192.168.20.1

## Switch0 Configuration

enable

configure terminal

vlan 10

name SALES

vlan 20

name ENG

interface fa0/1

switchport mode access

switchport access vlan 10

interface fa0/2

switchport mode access

switchport access vlan 20

interface gi0/1

switchport mode trunk

interface gi0/2

switchport mode trunk

end

copy running-config startup-config

## Switch1 Configuration

enable

configure terminal

vlan 10

name SALES

vlan 20

name ENG

interface fa0/1

switchport mode access

switchport access vlan 10

interface fa0/2

switchport mode access

switchport access vlan 20

interface gi0/1

switchport mode trunk

end

copy running-config startup-config

## Router1 Configuration (Router-on-a-Stick)

enable

configure terminal

interface g0/0/0

no shutdown

interface g0/0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

interface g0/0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

end

copy running-config startup-config

## Verification Commands

### Check VLANs

show vlan brief

### Check Trunk Ports

show interfaces trunk

### Check Router Interfaces

show ip interface brief

Expected:

Gi0/0/0      up/up

Gi0/0/0.10   up/up

Gi0/0/0.20   up/up

## Connectivity Tests

From PC01:

ping 192.168.10.1

ping 192.168.10.20

ping 192.168.20.10

From PC02:

ping 192.168.20.1

ping 192.168.20.20

ping 192.168.10.10

## Key Concepts

Access Port:

- Carries one VLAN

- Used for PCs

Trunk Port:

- Carries multiple VLANs

- Uses 802.1Q tagging

Router-on-a-Stick:

- One physical router interface

- Multiple subinterfaces

- Each subinterface serves one VLAN gateway

Traffic Flow:

PC → Access Port → VLAN → Trunk → Router → Trunk → VLAN → Destination